KNX 8-way switch actuator, basic device (RMG 8 S KNX)

EIB/KNX switching actuator with relay outputs for 8 switching channels; upgradeable with RME 8 S for up to 24 channels or with other MIX upgrade devices; rail mounting housing width 72 mm; basic device and bus module can be separated mechanically and can be swapped independently of each other. LED switching status display on device for each channel; manual switch with status display (also without bus voltage); links, type of contact (NC contact/NO contact) and participation in central commands such as continuous On, continuous Off, central switching and save/call up scene; switching functions: On/Off, impulse, On/Off delay, staircase light with warning according to DIN 18015-2; logical links: lock, AND, release, OR. Feedback object for each channel, central objects with and without priority, select and save scenes (object); scene object for every channel; switching capacity; 16A/250V~ cos phi = 1.

Deliver, assemble, connect up and set up

...

Make: Theben

Type: RMG 8 S KNX

Order no.: 4930220

Unit: ST